MSK Modulation & Demodulation

User Documentation

Python Signal Processing Suite

# 1. Overview

Minimum Shift Keying (MSK) is a continuous-phase frequency-shift keying (CPFSK) scheme widely used in digital communications — most notably in GSM mobile networks, GMSK variants, and satellite links. This suite provides a self-contained Python implementation for modulating, transmitting through a noisy channel, and coherently demodulating MSK signals, along with tools to measure and visualise performance.

## 1.1 How MSK Works

MSK encodes each binary bit as one of two sinusoidal frequencies:

- Bit 1  →  fₙₕᴵʰ = fᴄ + 1/(2T)

- Bit 0  →  fₗₒᵂ  = fᴄ − 1/(2T)

where fᴄ is the carrier frequency and T is the bit duration. The key property is continuous phase: the phase at the end of each bit is carried forward to the next, ensuring no abrupt discontinuities in the waveform. This gives MSK a compact spectrum compared to ordinary FSK.

## 1.2 Coherent Demodulation

The demodulator correlates each received bit segment against phase-coherent local references for both candidate frequencies. Because MSK has phase memory, the accumulated phase from all previous bits is tracked and applied to each reference. The bit decision is made by selecting whichever frequency produces the higher inner product with the received segment.

# 2. Requirements

The suite requires Python 3.8 or later and the following packages:

| Package | Purpose |
| --- | --- |
| numpy | Numerical arrays, FFT, random number generation |
| matplotlib | All plots: time-domain, spectrum, BER, I/Q constellation |

Install both with pip:

pip install numpy matplotlib

# 3. File Structure

| File | Role |
| --- | --- |
| msk.py | Core library. All signal processing functions live here. Import this module from your own scripts. |
| mod.py | Modulator demo. Generates an MSK signal from a fixed bit sequence and displays the time-domain waveform and frequency spectrum. |
| demod.py | Demodulator demo. Decodes both a clean and a noisy signal, prints the recovered bits, and shows a four-panel analysis figure (time domain, BER curve, two I/Q constellations). |

# 4. Running the Scripts

## 4.1 mod.py — Modulator Demo

Generates an MSK-modulated waveform from a hard-coded 7-bit sequence and displays two plots: the time-domain signal and the frequency spectrum.

Run from the project directory:

python mod.py

Expected output: a matplotlib window with two subplots.

- Top — Time Domain: the MSK waveform oscillating between the two frequencies over ~7 ms.

- Bottom — Frequency Spectrum: magnitude in dB vs. frequency (kHz), showing peaks at fᴄ ± 0.5 kHz (i.e., 1.5 kHz and 2.5 kHz).

## 4.2 demod.py — Demodulator Demo

Runs the full transmit-channel-receive chain, prints bit recovery results to the console, and displays a four-panel figure.

python demod.py

Console output example:

Original:           [1, 0, 1, 1, 0, 0, 1]

Decoded (no noise):  [1, 0, 1, 1, 0, 0, 1]

Decoded (SNR=10 dB): [1, 0, 1, 1, 0, 0, 1]

Four-panel figure:

- Top-left — Time Domain: clean signal overlaid with the noisy version at the demo SNR.

- Top-right — BER vs. SNR: simulated bit error rate (log scale) swept from 0 to 15 dB, using 1000 random bits per point.

- Bottom-left — I/Q Constellation at SNR = 5 dB: wide scatter showing noise impact.

- Bottom-right — I/Q Constellation at SNR = 15 dB: tighter cluster demonstrating clean recovery.

# 5. API Reference (msk.py)

All functions are in msk.py and can be imported directly:

from msk import msk_modulate, msk_demodulate, awgn, compute_ber, ber_vs_snr, iq_samples

## 5.1 msk_modulate

Generates an MSK-modulated signal with continuous phase from a list of binary bits.

### Signature

t, signal = msk_modulate(data, bit_rate, carrier_freq, sample_rate)

### Parameters

| Parameter | Type | Description |
| --- | --- | --- |
| data | list[int] | Binary bit sequence to modulate. Each element must be 0 or 1. |
| bit_rate | float | Bit rate in bits per second (e.g. 1e3 for 1 kbps). |
| carrier_freq | float | Centre carrier frequency in Hz (e.g. 2e3 for 2 kHz). |
| sample_rate | float | Number of samples per second (e.g. 1e5 for 100 kHz). Should be at least 10× the highest signal frequency. |

### Returns

- t — numpy.ndarray of time values in seconds, length = len(data) × (sample_rate / bit_rate).

- signal — numpy.ndarray of float amplitudes in the range [−1, +1].

### Example

from msk import msk_modulate

bits = [1, 0, 1, 1, 0]

t, sig = msk_modulate(bits, bit_rate=1e3, carrier_freq=2e3, sample_rate=1e5)

print(sig.shape)  # (500,)  — 100 samples per bit × 5 bits

## 5.2 awgn

Adds Additive White Gaussian Noise (AWGN) to a signal at a specified signal-to-noise ratio.

### Signature

noisy = awgn(signal, snr_db)

### Parameters

| Parameter | Type | Description |
| --- | --- | --- |
| signal | numpy.ndarray | Clean input signal. |
| snr_db | float | Desired signal-to-noise ratio in decibels. Higher values mean less noise. Typical test range: 0–20 dB. |

### Returns

- noisy — numpy.ndarray of the same shape as signal with Gaussian noise added.

### Example

from msk import msk_modulate, awgn

_, sig = msk_modulate([1,0,1], 1e3, 2e3, 1e5)

noisy = awgn(sig, snr_db=10)   # 10 dB SNR

## 5.3 msk_demodulate

Coherently demodulates an MSK signal, recovering the original bit sequence. Tracks accumulated phase across bits for correct decisions.

### Signature

bits = msk_demodulate(signal, bit_rate, carrier_freq, sample_rate)

### Parameters

| Parameter | Type | Description |
| --- | --- | --- |
| signal | numpy.ndarray | Received signal (clean or noisy). Must contain at least one full bit worth of samples. |
| bit_rate | float | Bit rate in bps. Must match the value used during modulation. |
| carrier_freq | float | Carrier frequency in Hz. Must match the value used during modulation. |
| sample_rate | float | Sample rate in samples/s. Must match the value used during modulation. |

### Returns

- bits — list[int] of recovered binary values (0 or 1), one per complete bit period found in the signal.

### Example

from msk import msk_modulate, msk_demodulate, awgn

data = [1, 0, 1, 1, 0, 0, 1]

_, sig = msk_modulate(data, 1e3, 2e3, 1e5)

noisy  = awgn(sig, snr_db=8)

decoded = msk_demodulate(noisy, 1e3, 2e3, 1e5)

print(decoded)  # e.g. [1, 0, 1, 1, 0, 0, 1]

## 5.4 compute_ber

Computes the Bit Error Rate (BER) between an original and a decoded bit sequence.

### Signature

ber = compute_ber(original, decoded)

### Parameters

| Parameter | Type | Description |
| --- | --- | --- |
| original | list[int] | Ground-truth bit sequence. |
| decoded | list[int] | Recovered bit sequence. If longer than original, the excess bits are ignored. |

### Returns

- ber — float in [0.0, 1.0]. 0.0 means perfect recovery; 1.0 means every bit is wrong.

### Example

from msk import compute_ber

ber = compute_ber([1,0,1,1], [1,0,0,1])

print(ber)  # 0.25  (one bit wrong out of four)

## 5.5 ber_vs_snr

Runs a Monte Carlo BER simulation over a sweep of SNR values using a fixed random bit sequence.

### Signature

snrs, bers = ber_vs_snr(bit_rate, carrier_freq, sample_rate, snr_range_db,

n_bits=1000, seed=42)

### Parameters

| Parameter | Type | Description |
| --- | --- | --- |
| bit_rate | float | Bit rate in bps. |
| carrier_freq | float | Carrier frequency in Hz. |
| sample_rate | float | Sample rate in samples/s. |
| snr_range_db | array-like | Sequence of SNR values (dB) at which to evaluate BER (e.g. numpy.arange(0, 16)). |
| n_bits | int (optional) | Number of random bits to use per SNR point. Default: 1000. Higher values give smoother curves. |
| seed | int (optional) | Random seed for reproducibility. Default: 42. |

### Returns

- snrs — the same snr_range_db passed in (returned for convenience when plotting).

- bers — list[float] of BER values, one per SNR point.

### Example

import numpy as np

import matplotlib.pyplot as plt

from msk import ber_vs_snr

snrs, bers = ber_vs_snr(1e3, 2e3, 1e5, np.arange(0, 16), n_bits=2000)

plt.semilogy(snrs, bers, marker='o')

plt.xlabel('SNR (dB)'); plt.ylabel('BER'); plt.grid(True)

plt.show()

## 5.6 iq_samples

Extracts one I/Q (in-phase / quadrature) sample per bit by mixing with carrier-frequency sinusoids and averaging over each bit period.

### Signature

I, Q = iq_samples(signal, carrier_freq, sample_rate, bit_rate)

### Parameters

| Parameter | Type | Description |
| --- | --- | --- |
| signal | numpy.ndarray | Received signal to analyse. |
| carrier_freq | float | Carrier frequency in Hz. |
| sample_rate | float | Sample rate in samples/s. |
| bit_rate | float | Bit rate in bps (defines the integration window). |

### Returns

- I — numpy.ndarray of in-phase values, length = number of complete bit periods.

- Q — numpy.ndarray of quadrature values, same length as I.

### Example

from msk import msk_modulate, iq_samples

import matplotlib.pyplot as plt

_, sig = msk_modulate([1,0,1,1,0,0,1], 1e3, 2e3, 1e5)

I, Q = iq_samples(sig, 2e3, 1e5, 1e3)

plt.scatter(I, Q); plt.xlabel('I'); plt.ylabel('Q')

plt.axis('equal'); plt.grid(True); plt.show()

# 6. Customising Parameters

All key parameters are defined at the top of mod.py and demod.py. Edit them directly before running.

| Variable | Default | Unit | Notes |
| --- | --- | --- | --- |
| binary_data | [1,0,1,1,0,0,1] | — | Any list of 0s and 1s. Longer sequences give better BER statistics. |
| bit_rate | 1e3 | bps | Increase for higher-speed simulation. Keep sample_rate ≥ 10× carrier frequency. |
| carrier_freq | 2e3 | Hz | Must satisfy carrier_freq > bit_rate/2 to avoid frequency overlap. |
| sample_rate | 1e5 | Hz | Must be at least 2× the highest frequency in the signal (Nyquist). Higher values improve accuracy. |
| snr_demo_db | 10 | dB | (demod.py only) SNR used for the time-domain overlay and bit-recovery printout. |

Important constraint: carrier_freq must be greater than bit_rate / 2. If they are too close, the two MSK frequencies (fᴄ ± 1/(2T)) will overlap or go negative.

To change parameters used in the BER curve, edit the ber_vs_snr call inside demod.py:

snr_range = np.arange(0, 21, 2)        # 0 to 20 dB in steps of 2

snrs, bers = ber_vs_snr(bit_rate, carrier_freq, sample_rate, snr_range, n_bits=5000)

# 7. Using msk.py in Your Own Code

msk.py is a plain Python module — place it in the same directory as your script and import it.

Complete end-to-end example:

import numpy as np

import matplotlib.pyplot as plt

from msk import msk_modulate, awgn, msk_demodulate, compute_ber, ber_vs_snr

# 1. Define parameters

BIT_RATE     = 1e3

CARRIER_FREQ = 2e3

SAMPLE_RATE  = 1e5

# 2. Modulate

data = [1, 0, 1, 1, 0, 0, 1, 0, 1, 0]

t, sig = msk_modulate(data, BIT_RATE, CARRIER_FREQ, SAMPLE_RATE)

# 3. Simulate channel at 8 dB SNR

received = awgn(sig, snr_db=8)

# 4. Demodulate

decoded = msk_demodulate(received, BIT_RATE, CARRIER_FREQ, SAMPLE_RATE)

print('BER:', compute_ber(data, decoded))

# 5. BER curve

snrs, bers = ber_vs_snr(BIT_RATE, CARRIER_FREQ, SAMPLE_RATE, np.arange(0,16))

plt.semilogy(snrs, bers, marker='o')

plt.xlabel('SNR (dB)'); plt.ylabel('BER')

plt.title('MSK BER Performance'); plt.grid(True); plt.show()

# 8. Troubleshooting

| Symptom | Fix |
| --- | --- |
| ModuleNotFoundError: msk | Ensure msk.py is in the same directory as the script you are running, or add its directory to PYTHONPATH. |
| No plot window appears | Some environments suppress matplotlib GUIs. Try adding import matplotlib; matplotlib.use('TkAgg') before other imports, or save figures with plt.savefig('out.png'). |
| Decoded bits are all wrong | carrier_freq or sample_rate mismatch between modulation and demodulation calls. Ensure all three system parameters are identical. |
| BER stays near 0.5 at all SNRs | bit_rate is too close to or greater than carrier_freq. Ensure carrier_freq > bit_rate / 2. |
| Slow BER simulation | Reduce n_bits in the ber_vs_snr call (default 1000 is fine for most uses) or narrow snr_range. |